Motive 3:

- Run as admin.

- Turn on Data Streaming for NatNet (not VRPN). The mocap machine should be on Ethernet not Wifi. Always turn off data streaming before quitting Motivate.

Context:

- The password for all root accounts on the ModalAI Sentinel drones is: oelinux123

- You will need to connect to PigeonLoft (LAN). Password: D76z@Pf#UYz9Xrr9

- When connecting to Sentinel 5 using QGC, use server address 192.168.168.110:14561 with port 14561 (yes, twice) to connect over WiFi.

- The Microhard radio’s address for Sentinel is 192.168.168.109, but you will not need this.

- The Wifi address for Sentinel 5 is 172.19.250.1:14561.

- The handheld RC controller that controls Sentinel 5 is labelled “192.”

- If in ADB or SSH and you cannot clone a repo, if you cannot clone a repo, try export GIT_SSL_NO_VERIFY=1.

- On the ros2humble_packages repo (Joe will need to give this to you), under bash-scripts, edit active_vehicles.env to enable topic publishing.

ADB:

- Adb devices

- Adb shell

Cd ros2humbe_packages/bash-scripts/run-gcs-discovery-server.sh

Then

Run ./run-mocap-server.sh

Run ./activate-mocap-server.sh

You cannot echo topics until you are a superclient because we use a discovery server.

source ~/ros2humble_packages/bash-scripts/gsc-superclient.sh

Ros2 topic list

If empty: ros2 daemen stop

Then try again. Running any ros2 command restarts the daemon.

Ros2 topic echo /rigid_bodies_ext

The message top ‘’ is invalid

Source ../aviary_messages_ws/install/setup.bash

Ros2 topic echo /rigid_bodies_ext

./run-odom-estimator.sh

Make sure odometry_estimator/config/*.yaml has a vehicle_name AND vehicle_ids. vehicle_name “sentinel5” and vehicle ids: [5]

In aviary_gcs_ws, px4_direct_publish: False.

With SSH or aDB shell, run

bash docker-scripts/run-ros-docker.sh

./docker-scripts/connect-to-ros-container.sh

Then run source hyrl/hyrl_ros_sources.sh

In ADB.

Source hurl

Then  ros2 launch ros2_mocap_client mocap_client.launch.py

SSH was

172.19.250.1

OUTSIDE docker in a new SSH session: hyrl_start_discovery_and_xrce.sh

TLDR:

TO RUN EXPERIMENT:

Terminal 1:

ssh root@172.19.250.1

bash docker-scripts/run-ros-docker.sh

Terminal 2:

ssh root@172.19.250.1

bash hyrl/hyrl_start_discovery_and_xrce.sh

Terminal 1 (Again):

source hyrl/hyrl_ros_sources.sh

ros2 launch ros2_mocap_client mocap_client.launch.py

Terminal 3

Run your code

Exiting:

./hyrl/hyrl_shutdown_discovery_and_xrce.sh

Ctrl+C the mocap node.